| Activity/Job Title | Whacker Packer Use – Compactor |  |  |  |  |  |  | Date |  |  | 23/04/2025 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Task/WBS Number | SWM-016 |  |  |  |  |  |  | Supplier |  |  | MEHCO - Team Engineering Services Pty Ltd |
| Location of Works |  |  |  |  |  |  |  | ABN |  |  | 97 089 427 010 |
| Competency / Certificates required: |  |  |  |  |  |  |  |  |  |  |  |
| Training: (e.g., Induction White Card) | General Construction Induction |  |  |  |  |  |  |  |  |  |  |
| PPE required: | ☐ | ☒ | ☐ | ☐ | ☒ | ☒ | Normal Requirements: Safety footwear, high visibility shirt or vest As Required Hearing protection, hand protection Provide UV sun protection where required, broad brimmed hat, UV rated clothing, SPF 50+ sunscreen, tinted safety glasses with adequate UV Protection |  |  |  |  |
| Minimum signage required |  |  |  |  |  |  |  |  |  |  |  |
| SWMS Prepared by | Cynthia Hegerty |  |  |  |  |  |  |  | Date | 23/04/2025 |  |
| Authorised by | Andrew Hegerty |  |  |  |  |  |  |  | Position | Director |  |
| Safety Notes |  |  |  |  |  |  |  |  |  |  |  |
| ALL PERSONS INVOLVED IN TASK MUST HAVE THIS SWMS COMMUNICATED TO THEM PRIOR TO WORK COMMENCING - Regular inspections and observations will be conducted by Site Supervisor/Project Management to ensure SWMS is being complied with. - Pre-Start will be undertaken to identify, control and communicate additional site hazards. - Work must cease immediately if incident or near miss occurs. SWMS must be amended in consultation with relevant persons. - Amendments must be approved by TEAM Project Manager and communicated to all affected workers before work resumes. - SWMS must be made available for inspection or review as required by WHS legislation. - Record of SWMS must be kept as required by WHS legislation (until job is complete or for 2 years if involved in a notifiable incident) |  |  |  |  |  |  |  |  |  |  |  |

| ID | Step | Possible Hazards | Initial Risk Score See Risk Matrix for Details | Control Measures Control measures have been identified using the Hierarchy of Control Elimination ->Substitution ->Engineering -> Admin -> PPE | Control Responsibility | Residual Risk Score See Risk Matrix for details |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | General Precautions | Hazardous atmosphere | High | - Ensure adequate ventilation in area when using fuel-powered equipment - Ensure good ventilation if using in deep excavations, trenches, etc | All workers | Medium |
|  |  | Fire / explosion | High | - Allow engine to cool before refuelling–avoid spilling fuel on hot parts | All workers | Low |
|  |  | Over-exertion/ strain injury | High | - Obtain assistance or use mechanical aids to lift machine into trenches, etc, or when unloading or loading off or onto vehicles. - Lift machine only by the specified lifting points to prevent damage or with a lifting device. | All workers | Medium |
| 2 | Pre-start checks | Loose parts | Medium | - Inspect machine and tighten loose bolts or screws, and check bolts on foot plate. - Ensure that all guards and covers are fitted over moving parts of machine. - Check operation of controls and switches, and ensure that fuel cap is secure. - Stand 4-stroke models up right for 2 minutes before starting after being laid down. | All workers | Low |
| 3 | Operation | Personal injury | High | - Persons operating compactor should wear close fitting work clothing that will allow free movement, eye and foot protection, and hearing protection. - Shut off engine before attempting to adjust, clean or service machine. - Do not alter engine speed–run engine only at speed specified by manufacturer. - Do not operate engine unless air cleaner if fitted and clean. - Use only correct fuel or fuel mixture to prevent damage to engine. ¦ Ensure adequate supply of fuel. - Grasp handles at corners of handle when operating– use “loose” grip to reduce stress and vibration on hands, wrists and arms from machine. ¦ Do not try to over power the rammer machine is designed to “walk” forward. - Always walk behind rammer, and allow machine to pull itself forward.¦ Do not stand in front of or pull rammer. - Operate so that shoe hits ground flat for best compaction, and not on toe or heel. - Reduce engine speed to SLOW if rammer needs to be lifted during operation. ¦ Do not attempt to compact deep layers– light layers will compact better. | All workers | Medium |
| 4 | Cleaning and maintenance | Personal Injury | High | - Switch machine OFF and allow to cool before working on the engine or any mechanical parts ¦ Do not open engine when hot - Clean dust and debris with stiff brush - Check all bolts, screws and connections, and inspect cable and fuel lines - Al repairs must be carried out by authorised person | All workers | Medium |

## Safe Work Method Statement - Work Team Sign-on/ Review Register

Personnel are required to sign this register to indicate they have read, understand and will adhere to the requirements of the SWMS

| This SWMS covers: |  |  |  | SWMS No: |  |
| --- | --- | --- | --- | --- | --- |
| Name | Employee Signature | Date | Name | Employee Signature | Date |

Risk Rating Matrix

Likelihood/ Probability

| What is the likelihood the situation or circumstance will happen? |  |  |  |  |
| --- | --- | --- | --- | --- |
| Level | Probability | Description | or the current process... | Frequency |
| A | Almost certain 80-100% | Is expected to occur in most circumstances. | cannot prevent this event, no alternate approaches or processes are available | Occurs 1 or more times per year |
| B | Likely 60-80% | Will probably occur in most circumstances. | cannot prevent this event, but a different approach or process might | Occurs between 1 and 3 years |
| C | Possible 40-60% | Might occur at some time. | may prevent this event, but additional actions will be required | Could happen between 3 and 10 years |
| D | Unlikely 20-40% | Could occur at some time. | is usually sufficient to prevent this type of event | Could happen between 10 and 30 years |
| E | Rare 0-20% | May occur in exceptional circumstances. | is sufficient to prevent this event | Highly unlikely that it will occur within 30 years |

Consequences/ Impact

|  | Insignificant | Minor | Moderate | Major | Catastrophic |
| --- | --- | --- | --- | --- | --- |
| Type | 1 | 2 | 3 | 4 | 5 |
| Technical | Minimal or no impact | Mod. reduction, same approach retained | Mod. reduction, but workarounds available | Major reduction, but workarounds available | Unacceptable, no alternatives exist |
| Schedule | Minimal or no impact | Additional activities required, able to meet needed dates | Milestones slip up to a month | Milestone slip of greater than a month or critical path impacted | Cannot achieve major program milestones |
| Cost | Minimal or no impact | 5%>0 budget change | 10>5% budget change | 15>10% budget change | >15% budget change |
| Harm to People - Safety | Near Miss | Injury requiring first aid | Injury requiring medical treatment | Lost Time Injury | Fatality and/or sever irreversible disability |
| Harm to People - Occupational Health | Exposure to health hazard resulting in temporary discomfort. | Exposure to health hazard resulting in temporary alterations/n limitations (no time lost). | Exposure to health hazards/agents (over the OEL) resulting in reversible impact on health (with time lost). | Exposure to health hazards/agents (significantly over the OEL) resulting in irreversible impact on health with loss of quality of life or single fatality. | Exposure to health hazards/agents (significantly over the OEL) resulting in irreversible impact on health with loss of quality of life of a numerous group/population or multiple fatalities. |
| Property Damage or Process Loss | < $ 2,000 | $2,000 to $10,000 | $10,000 to $50,000 | $50,000 to $250,000 | $250,000 + |
| Process Interruption Consequence | < 1 hour | 1 hour to 1 shift | 1 shift to 1 day | 1 day to 1 week | 1 week + |
| Environmental Impact | Lasting days or less. Limited to small area (metres) receptor of low significance/sensitivity (industrial area). | Lasting weeks. Reduced area (hundreds of metres). No environmentally sensitive species/habitat. | Lasting months. Impact on extended area (kilometres) area with some environmental sensitivity (scarce/valuable environment). | Lasting years. Impact on sub-basin. Environmentally sensitive environment/ receptor (endangers species/ habitats). | Permanent impact affects a whole basin or region. Highly sensitive environment (endangers species, wetlands, protected habitats). |
| Social/ Community Impact | Minor disturbance of culture/social structures. | Some impacts on local populations, mostly repairable. Single stakeholder complaint in reporting period. | Ongoing social issues. Isolated complaints from community members/stakeholders. | Significant social impacts. Organised community protests threatening continuity of operations. | Major widespread social impacts. Community reactions affecting business continuity. “Licence to operate” under jeopardy. |
| Legal and Regulatory | Technical non-compliance. No warning received. No regulatory reporting required. | Breach of regulatory requirements. Report/involvement of authority. Attracts administrative fine. | Minor breach of law. Report/investigation by authority. Attracts compensation/ penalties/ enforcement action. | Breach of law. May attract criminal prosecution of Operating Co. and/or of Directors/Managers and penalties/ enforcement action. Individual licence temporarily revoked. | Significant of the law. May attract Individual or class action law suits, criminal prosecution of Co. Directors/Managers. Suits against parent Co.; permit to operate substantially modified or withdrawn. |
| Reputation | Internal issues identified but manageable. | Minor impact, awareness/ concern from specific individuals. | Limited impact. Concern/ complaints from certain groups/ organisation (e.g. NGOs). | Local impact. Public concern/ adverse publicity localised within neighbouring communities. | Suspected reputation damage. ocal/ regional public concern and reactions. |

Risk Rating

| A | Medium | High | High | Extreme | Extreme |
| --- | --- | --- | --- | --- | --- |
| B | Medium | Medium | High | High | Extreme |
| C | Low | Medium | Medium | High | Extreme |
| D | Low | Low | Medium | Medium | High |
| E | Low | Low | Low | Medium | Medium |
|  | 1 Insignificant | 2 Minor | 3 Moderate | 4 Major | 5 Catastrophic |

Guidelines for approval

| Risk Level | Guidelines for risk matrix |
| --- | --- |
| Low | Approval to proceed must come from the Supervisor Monitor and manage as appropriate |
| Medium | Approval to proceed must come from the Project Manager Actively manage |
| High | Approval to proceed must come from the Operations Manager Proactively manage with systems and approval of same by SMT – must include improvements to decrease level of risk |
| Extreme | Approval to proceed must come from the Managing Director Eliminate, avoid, implement specific action plans/procedures to manage and monitor. |